Naturally Occurring Radioactive Materials (NORM)

Introduction
Naturally Occurring Radioactive Materials (NORM) are found in soil, water, and air. These materials contain radioactive elements such as uranium (U-238), thorium (Th-232), radium (Ra-226, Ra-228), and potassium-40 (K-40). In their natural state, NORM is not usually dangerous. However, when disturbed by human activities, radiation levels can rise and pose health risks.

Industries such as mining, oil and gas, water treatment, and construction often encounter NORM. If not managed properly, radiation exposure can be harmful. Long-term exposure to ionizing radiation may damage cells, increase the risk of cancer, and cause genetic mutations or organ damage. To prevent harm, agencies such as CNSC, the U.S. Nuclear Regulatory Commission (NRC), and the International Atomic Energy Agency (IAEA) provide safety guidelines. The ALARA principle (As Low As Reasonably Achievable) is key to minimizing exposure by limiting time, maintaining distance, and using shielding.

This document explains how NORM is classified, the health risks involved, exposure limits, and best safety practices. It follows Canadian, U.S., and international standards to ensure safe handling in workplaces and industries.

Types of Ionizing Radiation from NORM

NORM emits three types of ionizing radiation: alpha, beta, and gamma. Each type behaves differently and has different health effects (CNSC, 2023).

| Radiation | Characteristics | Health Risks |
| --- | --- | --- |
| Alpha Radiation | Alpha particles are large, heavy, and positively charged. They have low penetration power and can be blocked by paper or human skin. | If inhaled, ingested, or absorbed through cuts, alpha radiation can damage internal cells and increase the risk of cancer. |
| Beta Radiation | Beta particles are smaller and lighter than alpha particles. They can travel several meters in the air and can be blocked by plastic, glass, or aluminum. | Beta radiation can cause skin burns and internal harm if inhaled or ingested. |
| Gamma Radiation | Gamma rays are high-energy waves that penetrate most materials. Only dense materials like lead, concrete, or thick steel can block them. | Gamma radiation affects the entire body. It can damage DNA, increase cancer risk, and cause radiation sickness. |

Health Effects of NORM Exposure

Exposure to ionizing radiation from NORM can be harmful. The effects depend on the radiation type, dose, and exposure duration. High doses cause immediate health issues, while long-term exposure may increase cancer risk.

Acute Health Effects

Radiation Sickness: A high radiation dose in a short time can cause acute radiation syndrome (ARS). Symptoms include nausea, vomiting, fatigue, and skin burns. Severe cases can be fatal.

Chronic Health Effects

Cancer Risk: Prolonged exposure increases the risk of leukemia, lung cancer, and skin cancer. Higher doses lead to greater risks.

Genetic Effects: Radiation can damage DNA, leading to genetic mutations that may affect future generations.

Radiation Exposure Limits

Regulatory agencies set dose limits to protect workers and the public. These limits help reduce radiation risks.

Canadian NORM Guidelines

Canada has strict rules for managing NORM exposure:

- Annual Dose Limit for Workers: 1 millisievert (mSv) per year.

- Equivalent Hourly Dose: Over a 2,000-hour work year, this equals 0.5 microsieverts per hour (µSv/hr).

If exposure is expected to exceed 0.3 mSv per year (0.15 µSv/hr), a site-specific risk assessment is required.

Annual Allowable Hours Based on Dose Rate

The table below shows the recommended and maximum allowable work hours at different dose rates:

| Dose Rate (µSv/hr) | Recommended Annual Hours | Max Annual Hours |
| --- | --- | --- |
| 0.5 | 600 | 2,000 |
| 1 | 300 | 1,000 |
| 2 | 150 | 500 |
| 3 | 100 | 333 |
| 4 | 75 | 250 |
| 5 | 60 | 200 |
| 6 | 50 | 167 |
| 7 | 43 | 143 |
| 8 | 38 | 125 |

NORM Contamination Classification

NORM contamination is categorized by radioactive concentration and surface radiation levels to determine handling and disposal measures.

| Category | Radionuclide Concentration (Bq/g) | Surface Radiation Level (cpm) |
| --- | --- | --- |
| Unrestricted | 0.3 – 10 | <2× background |
| NORM-Contaminated | 10 – 70 | 2× background – 2,000 |
| Radioactive | >70 | >2,000 |

Safety Precautions for Handling NORM

Following safety guidelines can help reduce exposure risks.

Radiation Exposure and Dose Limits

To protect workers and the public, regulatory agencies set restrictions on radiation exposure. The aim of these rules to minimize health risks linked to ionizing radiation.

The table below presents the maximum radiation dose permitted for different categories:

| Category | Annual Dose Limit |
| --- | --- |
| Occupational Worker | 50 mSv (Canada, USA - NRC) |
| Public Exposure | 1 mSv |
| Radon Progeny | 4 pCi/L (EPA) |

If someone is exposed to more than 0.3 mSv annually, a detailed risk assessment and radiation monitoring program must be carried out.

Detecting and Measuring NORM

Specialized instruments are needed to detect and measure Naturally Occurring Radioactive Materials (NORM). These tools help track radiation levels in workplaces, industrial sites, and the environment. Each device has its own level of sensitivity, detection range, and ideal use (Canadian Nuclear Safety Commission [CNSC], 2023).

Geiger-Müller (GM) counter

A Geiger-Müller (GM) counter is a commonly used instrument for detecting radiation, especially beta and gamma rays. It operates within a detection range of 0.01 to 1000 µSv/hr, making it useful for a variety of radiation monitoring tasks. These counters work well for high-energy beta and gamma radiation, though they are less effective in picking up alpha particles. Due to their quick response time and ease of use, they are widely utilized in industrial radiation monitoring, workplace safety checks, and environmental surveys.

Scintillation Detectors

When measuring radiation, some detectors pick up electrical signals, while others, like scintillation detectors, work in a different way. They use a special material that glows when it comes into contact with radiation. The amount of light produced tells us how much radiation is present. These detectors are highly sensitive and can detect even the weakest levels—as low as 0.001 µSv/hr. They are especially useful for picking up radiation that other detectors might miss, making them a common choice for checking contamination, monitoring the environment, and scanning in medical settings.

Gamma Spectroscopy

Gamma spectroscopy is used to study gamma radiation and find out which radioactive elements are present in a sample. This method measures radiation activity, which can range from 0.1 Bq/kg to several kBq/kg, depending on the type of material being tested.

It is highly perfect in detecting uranium (U-238), thorium (Th-232), radium (Ra-226), and potassium-40 (K-40). It is commonly used in mining, oil and gas operations, environmental monitoring, and laboratory testing.

Comparison of NORM Detection Methods

| Detection Method | Radiation Type Detected | Sensitivity Level | Common Uses |
| --- | --- | --- | --- |
| Geiger-Müller Counter | Beta, Gamma | Moderate | Field surveys, workplace radiation monitoring |
| Scintillation Detector | Alpha, Beta, Gamma | High | Environmental assessments, contamination checks |
| Gamma Spectroscopy | Gamma | Very High | Identifying radionuclides in laboratory settings |

Managing and Disposing of NORM Waste

Safe disposal of Naturally Occurring Radioactive Material (NORM) waste helps protect people and the environment. The amount of radiation in the waste decides how it should be handled. It must follow strict rules set by the Canadian Nuclear Safety Commission (CNSC), U.S. Nuclear Regulatory Commission (NRC), and Environmental Protection Agency (EPA). To ensure safety, different organizations set rules for handling and disposing of NORM waste. These include the Canadian Nuclear Safety Commission (CNSC), U.S. Nuclear Regulatory Commission (NRC), and the Environmental Protection Agency (EPA). Their goal is to limit radiation exposure and prevent contamination.

Types of NORM Waste

NORM waste is divided into three categories based on how much radiation it contains.

- Low-Activity Waste (< 10 Bq/g): If radiation levels are below a certain threshold, this waste can be disposed of in regular landfills, as long as it meets clearance levels.

- Medium-Activity Waste (10 - 70 Bq/g): This type requires disposal in special landfills that have protective barriers to keep radiation from seeping into groundwater. Containers must be sealed, and the waste must go through radiation testing before disposal.

- High-Activity Waste (> 70 Bq/g): Since this waste poses a higher risk, it must be processed at licensed radioactive waste facilities. Some materials may even need long-term storage deep underground to prevent radiation exposure.

| Waste Type | Radioactivity Level | Disposal Method |
| --- | --- | --- |
| Low-Activity Waste | < 10 Bq/g | Standard landfill disposal (below regulatory limits) |
| Medium-Activity Waste | 10 - 70 Bq/g | Controlled landfill disposal (requires conditions) |
| High-Activity Waste | > 70 Bq/g | Radioactive waste facility processing |

Getting Rid of NORM Waste

The way NORM waste is handled depends on how much radiation it has.

Low Radiation Waste

- It can go to normal landfills if it meets safety rules.

- Government agencies check that it will not cause harm.

Medium Radiation Waste

- It must go to special landfills that have extra protection.

- The waste is sealed and tested to make sure it does not leak.

High- Radiation Waste

- Sent to licensed facilities that process and store it safely.

- Treated using solidification, encapsulation, and secure containment to prevent leaks.

- Some highly radioactive materials must be placed in deep underground storage for long-term safety.

Regulations for NORM Waste Disposal

Different agencies have rules to keep people and the environment safe when dealing with NORM waste.

- CNSC (Canada): Makes rules for safe disposal.

- NRC (USA): Uses 10 CFR Part 20 to protect workers and the public from radiation.

- EPA (USA): Sets environmental safety rules for handling radioactive waste.

- IAEA (Global): Creates international standards for safe NORM disposal.

Facilities that manage NORM waste must follow safety steps. They need to check radiation levels, train workers, and reduce environmental risks (IAEA, 2023).

Responding to NORM Emergencies

If NORM waste spills or leaks, it must be handled fast to prevent harm. Without proper action, radiation could spread, putting workers at risk. To stay safe, workplaces that deal with NORM need a solid emergency plan to control the situation (Canadian Nuclear Safety Commission [CNSC], 2023).

What Happens in an Emergency?

The first priority is to get everyone away from danger. People near the area should leave right away. A restricted zone is then set up to stop others from entering. At the same time, radiation levels are measured to assess the risk.
After clearing the area, the next focus is containing the spill. Workers place barriers, absorbent materials, and shielding to stop further spread. If there is a risk of radioactive dust in the air, ventilation systems might be shut down to stop it from moving around.

If a person is exposed, decontamination starts immediately. They must take off any contaminated clothing, which is then sealed in a safe container. Their skin is washed well with soap and water to remove any remaining radiation. Afterward, radiation levels are checked again to ensure they are no longer at risk.

Radiation monitoring is also performed on the affected area using Geiger counters and scintillation detectors. This helps responders figure out if the cleanup is complete or if further action is needed. Contaminated materials are gathered, labeled, and disposed of according to NORM waste regulations. A detailed report is then sent to the proper regulatory agency. This includes information about what happened, how the situation was handled, and any further steps needed.

Keeping Workers Safe

- Respiratory protection is a must. Workers use P100 respirators to avoid inhaling radioactive dust.

- Protective clothing such as gloves, coveralls, and goggles help keep skin and eyes safe.

- Radiation checks are done on every worker before they leave the site to ensure no contamination has spread.

Wearing the Right Gear When Working with NORM

If you’re handling Naturally Occurring Radioactive Materials (NORM), the right protective gear is just as important as knowing what you’re dealing with. You can’t always see or feel radiation, but that doesn’t mean it’s not there. Long-term exposure can build up over time, so protective equipment isn’t optional—it’s necessary.

But here’s the thing: not all NORM situations require the same level of protection. A worker doing routine monitoring in a low-risk area won’t need the same gear as someone handling high-radioactivity waste. The type of Personal Protective Equipment (PPE) depends on what kind of exposure you’re dealing with.

The Basics: What Workers Wear and Why

Some jobs need full protective suits, while others might just require gloves and safety glasses. It all depends on how much direct contact there is with radioactive material.

Respirators – Used in areas where NORM particles can become airborne. A P100 respirator works for low-risk situations, but when there’s a lot of dust in the air, workers need full-face respirators.

Gloves – Since hands touch everything, they need protection. Nitrile or neoprene gloves block radioactive dust and contaminants from making direct contact with the skin.

Safety Glasses or Goggles – Prevent radiation dust from getting into workers' eyes. Simple safety glasses work for some environments, but in high-contamination zones, sealed goggles are better.

Protective Clothing – Workers in areas with higher radiation exposure wear coveralls or disposable suits to keep radioactive particles off their regular clothes.

Boot Covers – Since radioactive particles settle on surfaces, they can stick to shoes. Workers in higher-risk areas wear rubber boots or disposable covers to avoid spreading contamination.

Why PPE Matters More Than You Think

One of the biggest mistakes people make is assuming PPE is just for extreme cases. Radiation exposure isn’t instant—it builds up. Even small amounts, over time, can add up to significant exposure. That’s why wearing the right protective equipment every time matters.

When used correctly, PPE helps:

- Prevent inhalation of radioactive dust

- Block direct skin contact with NORM materials

- Stop contamination from spreading to other areas

How to Use PPE the Right Way

Wearing PPE isn’t enough—you have to use it correctly to actually stay safe.

Check everything before starting work. A cracked face shield or a torn glove won’t do much good.

Don’t remove PPE in random areas. Taking off protective gear in the wrong place can spread contamination.

Dispose of used PPE properly. Contaminated gear isn’t regular trash—it needs to be handled as radioactive waste.

The best way to stay safe when working around NORM? Don’t take chances. PPE is only part of the equation—proper handling, training, and exposure monitoring all play a role in keeping workers protected. The right gear used the right way makes all the difference.

Conclusion

Managing NORM requires strict safety measures. Agencies in Canada, the U.S., and international groups set rules to protect workers. The ALARA principle (As Low As Reasonably Achievable) helps reduce exposure. This includes safety controls, PPE, monitoring, and proper disposal. Following these guidelines ensures safety and legal compliance.

For more information, refer to:

CNSC Regulations:https://www.cnsc-ccsn.gc.ca/eng/acts-and-regulations/regulations/ 
NRC 10 CFR Part 20: https://www.ecfr.gov/current/title-10/chapter-I/part-20?toc=1 
IAEA Safety Standards: https://www-ns.iaea.org/downloads/standards/iaea-safety-standards-brochure.pdf

Sources Used:

https://www.cnsc-ccsn.gc.ca/eng/resources/research/waste-management/

https://www.cnsc-ccsn.gc.ca/eng/resources/research/radiation-protection/

https://www.canada.ca/en/health-canada/services/publications/health-risks-safety/canadian-guidelines-management-naturally-occurring-radioactive-materials.html

https://www.epa.gov/radiation/radiation-health-effects

https://www.iaea.org/topics/radiation-safety-norm

https://www.iaea.org/publications/search/type/safety-standards-series

https://www.iaea.org/topics/emergency-preparedness-and-response-epr

https://www.iaea.org/publications/6482/management-of-radioactive-waste-from-the-mining-and-milling-of-ores

https://www.osha.gov/ionizing-radiation/health-effects

https://www.energy.gov/sites/default/files/2024-12/2023%20ORER%20-%20EHSS-2024-000533_final.pdf

https://www.who.int/news-room/fact-sheets/detail/ionizing-radiation-and-health-effects

https://www.nrc.gov/about-nrc/radiation.html

https://www.nrc.gov/waste.html

https://www.cnsc-ccsn.gc.ca/eng/resources/research/radiation-protection/

https://www.epa.gov/radiation/radiation-health-effects

https://www.energy.gov/

https://www.iaea.org/topics/radiation-safety-norm

https://www.who.int/news-room/fact-sheets/detail/ionizing-radiation-and-health-effects

https://www.iaea.org/topics/radiation-safety-norm

https://www.who.int/news-room/fact-sheets/detail/ionizing-radiation-and-health-effects

https://www.cnsc-ccsn.gc.ca/eng/resources/research/radiation-protection/

https://www.who.int/news-room/fact-sheets/detail/ionizing-radiation-and-health-effects

https://www.cnsc-ccsn.gc.ca/eng/resources/research/radiation-protection/

https://www.epa.gov/radiation/radiation-health-effects

https://www.who.int/news-room/fact-sheets/detail/ionizing-radiation-and-health-effects

https://www.iaea.org/topics/emergency-preparedness-and-response-epr

https://www.cnsc-ccsn.gc.ca/eng/resources/research/radiation-protection/

https://www.cnsc-ccsn.gc.ca/eng/resources/research/radiation-protection/

https://www.cnsc-ccsn.gc.ca/eng/resources/research/radiation-protection/

https://www.epa.gov/radiation/radiation-health-effects

https://www.cnsc-ccsn.gc.ca/eng/resources/research/radiation-protection/

https://www.cnsc-ccsn.gc.ca/eng/resources/research/radiation-protection/

https://www.epa.gov/radiation/radiation-health-effects

https://www.cnsc-ccsn.gc.ca/eng/resources/research/waste-management/

https://www.epa.gov/radiation/radiation-health-effects

https://www.nrc.gov/about-nrc/radiation.html

https://www.cnsc-ccsn.gc.ca/eng/resources/research/waste-management/

https://www.nrc.gov/about-nrc/radiation.html

https://www.nrc.gov/about-nrc/radiation.html

https://www.nrc.gov/about-nrc/radiation.html

https://www.cnsc-ccsn.gc.ca/eng/resources/research/waste-management/

https://www.nrc.gov/waste.html

https://www.nrc.gov/waste.html

https://www.energy.gov/sites/default/files/2024-12/2023%20ORER%20-%20EHSS-2024-000533_final.pdf

https://www.iaea.org/publications/6482/management-of-radioactive-waste-from-the-mining-and-milling-of-ores

https://www.iaea.org/publications/search/type/safety-standards-series

https://www.osha.gov/ionizing-radiation/health-effects

https://www.canada.ca/en/health-canada/services/publications/health-risks-safety/canadian-guidelines-management-naturally-occurring-radioactive-materials.html

https://www.canada.ca/en/health-canada/services/publications/health-risks-safety/canadian-guidelines-management-naturally-occurring-radioactive-materials.html

https://www.epa.gov/radiation/radiation-health-effects

https://www.nrc.gov/waste.html

https://www.canada.ca/en/health-canada/services/publications/health-risks-safety/canadian-guidelines-management-naturally-occurring-radioactive-materials.html

https://www.epa.gov/radiation/radiation-health-effects

https://www.nrc.gov/waste.html

https://www.epa.gov/radiation/radiation-health-effects